SNMPc 5 Network Management Platform

DDE Programming Guide

August 2001

Castle Rock Computing

12930 Saratoga Ave

Saratoga, CA 95070

Phone:	(408) 366-6540

Fax:	(408) 252-2379

Email:	sales@castlerock.com

WEB: 	www.castlerock.com

# Table Of Contents

Table Of Contents	2

Introduction	3

DDE Message Overview	3

SNMP Variable Name Format	4

Using DDE Request/Poke	5

AItem Formats	5

General Data Format	5

Set Data Format	5

Get/Getnext Data Format for MIB Variables	6

Getnext Data Format for MIB Tables	6

Get/Getnext Data Format for Pseudo-Variables	7

Pseudo-variables	7

Using DDE Execute	11

Command Format	11

Dialog Commands	11

MIB Display Commands	12

Program Commands	12

SNMP Commands	13

Command Examples	14

DDE Errors	14

DDELINK DLL Interface	15

Overview	15

SNMPC_INIT	15

SNMPC_CLOSE	15

SNMPC_EXEC	16

SNMPC_POKE	17

SNMPC_REQUEST	18

# Introduction

SNMPc 5 supports part of the SNMPc 4.x DDE application programming interface.  This document defines the low-level DDE commands and also the DDELINK high level DLL for using the DDE commands.

# DDE Message Overview

The supported DDE commands are summarized in the following table.

| Message | Description |
| --- | --- |
| INITIATE | Start up conversation with SNMPc. Set Application atom to SNMPc. Set Topic atom to SNMP-API. |
| REQUEST | Perform a GET or GETNEXT request. AItem contains command, node name, and object identifier. Variable and value are returned in WM_DDE_DATA message. |
| POKE | Perform a SET request. AItem atom contains command, node name, and object identifier; Data contains value. |
| EXECUTE | Execute an SNMPc command, or perform a vectored (multi-variable) SNMP GET, SET, or GETNEXT operation. The aitem atom is not used. The command is passed in a shared memory block. Requested variable and value list returned in WM_DDE_DATA message. Data returning commands are not available if you use the DDEML library of Visual Basic programs. |
| TERMINATE | Stop a conversation with SNMPc. |

# SNMP Variable Name Format

SNMP Variables specified in any of the DDE SNMP operations have the following format:

address @ community : mib-name | variable . instance

Spaces are shown above for clarity only - there are no spaces in the variable.

address @ is an optional IP or IPX address which overides the SNMPc node address.  This feature can be used when accessing multiple agents represented by a single SNMPc icon.   An example is 198.92.129.1@ifInOctets.1.

community : is an optional community name which overides the SNMPc node community name.  This feature can be used when accessing different community domains within the same agent, such as in hubs that use different community names for multiple hub entities in the same device.   An example is card1:ifInOctets.1.

mib-name | specifies the mib name where the variable is defined.  It is important to always specify the mib name to improve performance.  If the mib name is not specified SNMPc has to search every mib for the variable name.   An example if RFC-1213|ifInOctets.1.

variable specifies the mib variable name, which can be a text name or dot notation, or a combination of both.   For a table name use the SEQUENCE definition name. Examples are ifInOctets and IfEntry.

For WM_DDE_REQUEST and WM_DDE_POKE (GET, GETNEXT, SET) the variable can also be an SNMPc pseudo-variable.  Pseudo-variables are prefixed by a dollar ($) character.

instance specifies the instance within a table, such as the port number for the IfTable.

When specifying multiple variables at the same time, the address and community name overide of the first variable is used for all variables in the list.

# Using DDE Request/Poke

## AItem Formats

The aItem atom is set to an SNMPc command string in both the REQUEST (get, getnext) and POKE (set) DDE commands. The aItem atom has the following format.

command, node-name, object-name , count

command is either get, set, or getnext and must be in lower case. Only getnext is available for table retrieval.  Use WM_DDE_REQUEST for get and getnext commands.  Use WM_DDE_POKE for set commands.

node-name is the optional SNMPc Node Name of the SNMP Agent that should receive the request.

object-name is any SNMP variable name (e.g. ifStatus.1) or any SNMPc MIB table name. MIB table names may also have an instance identifier. For example, IfEntry.2 refers to the second entry in the interfaces table. MIB table names are specified in SEQUENCE definitions in the MIB source files, and appear in the Display MIB table and Edit MIB table dialog boxes.   object-name can also be one of several pseudo-variables, which are defined in a following section.

count is an optional argument that is only used for table names. It specifies how many table entries to retrieve.

The command, node-name, object-name and count strings are separated by a comma. There are no spaces present. The whole string is null terminated.

In all cases, the aItem atom sent in commands is reflected in the DATA or ACK response from SNMPc. You must delete the returned atom.

## General Data Format

The data sent with POKE messages or received with DATA messages is in CF_TEXT format. Variable values are always ASCII strings and are in the same format as used by the SNMPc Edit MIB Table and Edit MIB Vars commands.

## Set Data Format

For the set command, the data must contain a single string representing the new value for the specified variable.

## Get/Getnext Data Format for MIB Variables

Data returned for the get and getnext commands contains the following four tab separated values:

1.	Number of  table entries retrieved (always 1)

2.	Number of  different variables per entry (always 1)

3.	The (possibly updated) variable name

4.	The variable value

The data is null terminated.

For  "get,bridge1,sysDescr.0", the data might contain:

1 <TAB> 1 <TAB> sysDescr.0 <TAB> bridge/router <NULL>

For  "getnext,bridge1,ifIndex.1"  the data might contain:

1 <TAB> 1 <TAB> ifIndex.2 <TAB> 2 <NULL>

## Getnext Data Format for MIB Tables

For MIB tables, only the getnext operation is permitted. The returned data is a tab separated list which contains the following sets of values:

1.	The number of table entries (rows) retrieved

2.	The number of variables (columns) in each entry

3.	The updated table name

4.	A tab separated list of variable names (columns names)

5.	A tab separated list of variable values (rows X columns values)

The following is an example of returned data for the Internet Address table, IpAddrEntry. Note that spaces and newlines in this example are for presentation purposes only.

2 <TAB> 5 <TAB> IpAddrEntry.128.9.118.88 <TAB>

ipAdEntAddr <TAB> ipAdEntIfIndex <TAB> ipAdEntNetMask <TAB>

ipEntBcastAddr <TAB> ipAdEntReasmMaxSiz <TAB>

128.6.1.1 <TAB> 255.255.0.0 <TAB> 3 <TAB> 1 <TAB> 1500 <TAB>

128.9.118.88 <TAB> 2 <TAB> 255.255.0.0 <TAB> 1 <TAB> 1500 <NULL>

Note that the returned table name contains the instance part of the last entry retrieved. You can use this to perform a sequence of getnext operations to retrieve a large table. SNMPc can only retrieve 8196 bytes of data in each DATA response.

If you do not specify the table instance part, the first table entry will be retrieved. If you specify an instance, the entry following the one specified will be retrieved. For example, if you specify IfEntry.1, you will get IfEntry.2. This is a function of the getnext operation.

You can use the optional count argument to specify how many table entries should be retrieved. The default is to retrieve all entries, but keep in mind that some tables (e.g. RMON tables) can be very large and SNMPc will only return up to 8196 bytes of data.

## Get/Getnext Data Format for Pseudo-Variables

The returned data is a tab separated list containing the following elements:

1.	Number of  entries (rows) received

2.	Number of  values (columns) per entry

3.	The (possibly updated) pseudo-variable name

4.	A tab-separated list of pseudo-variable values (rows X columns values)

## Pseudo-variables

Each of the available pseudo-variables is described below, along with the allowable commands, and the instance and returned data formats, if any.

#### $error - Get last DDE error

get,,$error

The data contains the SNMPc error number of the last failed DDE operation for this client.  This variable should be used by Visual Basic programs because VB does not return the error status of failed operations.  Note that the error is not reset when an operation  succeeds.

#### $mapint - Map enumerated integers to numbers

set,,$mapint.0

The data is "on" to enable integer name mapping, or "off" to disable mapping.  When disabled, integer values, such as ifAdminStatus are returned as numbers instead of "up" or "down".

#### $mib - Get MIB Database information

get,,$mib.object-id

getnext,,$mib.object-id

'object-id' is any SNMP Object Identifier (OID) or an SNMPc table

name (SEQUENCE). It can be in text format, dot format, or a

combination of both. Valid examples are: iso, sysDescr,

ifIndex.2, 1.1.1.3, IfEntry, SystemInfo

Use the Get operation to get information about the named object. Use the Getnext operation to get information about the next object in the mib. If you specify a table name, getnext gets the next table.  If you specify a leaf object, getnext gets the next leaf object.

The returned data contains 5 columns. Each column is described below:

1.  The text format of the OID  (e.g. sysDescr)

2.  The dot format of the OID (e.g. 1.3.6....)

3.  The object type (in hex); one of the following:

Integer		2

Octet String		4

Null			5

Object ID		6

Display String	AA

IP Address		40

Counter		41

Gauge		42

Time Ticks		43

Opaque		44

4.  The allowed access; one of the following:

none		0

read only		1

read write		2

no access		4

obsolete		5 (no support in SNMPc)

optional		6 (no support in SNMPc)

5.  A non-zero value if the OID is an index, a 0 otherwise

In the case of a table (SEQUENCE name), there will be one row containing the above values for each variable in the table.

When you specify a leaf object name there will be only one row returned.  For table (i.e., SEQUENCE) names, there is one row returned for each element of the table.  For example, for the SystemInfo table, there will be one row returned for each of the sysDescr, sysObjectID, sysUpTime, and so on.

#### $mibExt  - Get extended MIB Database information

get,,$mibExt.[op2].object-id

getnext,,$mibExt.object-id

‘[op2]’ is an optional secondary command. It is one of the following:

[prevsib]		- get the previous sibling

[nextsib]		- get the next sibling

[parent]		- get the parent

[child]		- get the first child

'object-id' is any SNMP Object Identifier (OID) or an SNMPc table

name (SEQUENCE). It can be in text format, dot format, or a

combination of both. Valid examples are: iso, sysDescr,

ifIndex.2, 1.1.1.3, IfEntry, SystemInfo

The returned data is the same as the $mib variable, with the following additional values:

6.  The number of enumerated types for integer variables

7.  The variable description (up to 4096 bytes

#### $mibenum - Get MIB variable enumerated values

get,,$mibenum.object-id

'object-id' is any INTEGER type SNMP Object Identifier (OID) It can

be in text format, dot format, or a combination of both.

The returned data contains one row for each enumeration. Each row has two columns, as follows:

1.  The text name of an enumeration

2.  The corresponding number in decimal

#### $nodebynameExt  - Get node information by name

get,,$nodebynameExt.node-name

getnext,,$nodebynameExt.node-name

'node-name' is the SNMPc map Node Name.

You can use the getnext operation to return information for all nodes in the map. Use $nodebynameExt.0 to get the first entry.  For the getnext operation the returned atom is updated to the next node-name.

The returned data contains 6 elements:

1.    node name

2.    IP/IPX address

3.    status of "up" or "down"

4.    value of Node Identify variable (or 'Hierarchy' , ‘Ping’, or 'Goto')

5.    name of exec file

6.    name of parent hierarchy

7.    mac address of first port

8.    icon name

9.    get community

10.  set community

11.  trap community

12.  description

13.  reserved

For values that are not set or applicable, "(null)" is returned.

#### $nodebyaddrExt - Get node information by address

get,,$nodebyaddrExt.a.b.c.d

'a.b.c.d' is an internet address, in dot notation.

The return value is the same as the $nodebynameExt variable.

#### $nodebymacExt - Get node information by MAC address

get,,$nodebymacExt.a.b.c.d.e.f

'a.b.c.d.e.f' is a mac address

The return value is the same as the $nodebynameExt variable.

#### $selnodes - Get list of selected nodes

get,,$selnodes.0

The returned data is a list of selected node names.

# Using DDE Execute

## Command Format

The DDE Execute message can be used to start up SNMPc commands, or to perform vectored SNMP requests.

The Aitem atom is not used.

The Exec command is passed in a shared buffer, and has the following format:

[operation(node,var0,var1,var2,.....varN)]

operation is a command to perform, which can be one of: get, gextnext, graph, chart, list, table, edit, run, macro, history, or set. These commands are described further below.

node is the SNMPc Node Name. This name must be present in the SNMPc map.

var0..varN are optional arguments. The number and format depend on the command selected, but generally the first argument is an SNMP table name.

In all cases, the command buffer is returned with a DDE ack response, and it must be deleted by the client.  For some operations a data buffer or a second negative ACK response will be returned after the command is completed.

## Dialog Commands

#### setvar - Edit MIB Vars

[setvar(node-name)]

The setvar command displays the SNMPc Edit MIB Vars dialog box.

#### edit - Edit MIB Table

[edit(node-name, table-name)]

The edit command displays the SNMPc Edit MIB Table dialog box. The first argument specifies an SNMP table name, with instance (e.g., IfEntry.1).

## MIB Display Commands

#### graph - Display a MIB table entry as a graph

[graph(node-name, table-name)]

The graph command opens one or more graph windows in SNMPc.  The first argument specifies a table entry name, such as IfEntry.1.

#### chart - Display a MIB table entry as a chart

[chart(node-name, table-name)]

The chart command opens a chart window in SNMPc.  The first argument specifies a table entry name, such as IfEntry.1.

#### table - Display a MIB table

[table(node-name, table-name)]

The table command opens a Display MIB Table window in SNMPc.  The first argument specifies a table name, such as IfEntry, optionally including an instance part.

## Program Commands

#### run - Run an SNMPc API program

[run(node-name, program-name, optional-argument)]

The run command starts an API program or macro file which resides in the c:/snmpc/apiexec directory. The first argument specifies the program name, which must not include a path. The second argument is an optional argument that is passed as the third argument to the API program. SNMPc will always pass the node name and SNMPc frame window number as the first and second arguments to an API program.

#### wrun - Run a Windows program

[wrun(node-name, program-name, optional-argument)]

The wrun command runs any Windows program.  The first argument is the file name of the program, and the second argument is a double quoted string containing any program arguments. The node-name is passed to the program, but the SNMPc window number is not.

## SNMP Commands

#### set_asy - Set multiple SNMP variables

[set_asy(node-name, var0, val0, var1, val2, ..., varN, valN)]

The set_asy command performs a multiple variable SNMP set operation. The arguments are a list of variable and value pairs. The values must have the correct format for the variable, as used by the Edit MIB Table and Edit MIB Variables dialog boxes. The values can be enclosed in double quotes if they includes spaces or commas. SNMPc does not  ask for confirmation for this operation.

#### get and getnext - Get multiple SNMP variables

[get(node-name, var0, var1, ..., varN)]

The get and getnext commands perform vectored versions of the SNMP get and getnext operations. The arguments specify a list of SNMP variable names. Each variable must include an instance part for the get operation, and may include a MIB name prefix for either operation.

The get and getnext operations return a positive ACK message when the message is interpreted and started. The operation is then performed asynchronously. When the operation is complete, an unsolicited WM_DDE_DATA message is sent to the client with the requested variable values.  Since this is a non-standard usage of the WM_DDE_EXEC message, these commands can not be used with the DDEML library or with Visual Basic.  To use these commands you must specify SNMP-API as the Topic atom when connecting to SNMPc.

The returned data buffer is a tab separated and null terminated list of variable/value pairs that has the following format.

1 <tab> count <tab> $oidvec <tab> var0 <tab> val0 <tab>

var1 <tab> val1 <tab> .... <varN> <tab> valN <null>

The second element contains the number of pairs returned. The first and third elements are fixed and always set to 1 and $oidvec, respectively. The $oidvec value allows a program to differentiate this data format from the data formats used by the WM_DDE_REQUEST response.

The variable/value pairs contain the results of the requested operation. Variable is the SNMP MIB variable name and Value is the value retrieved from the node for the variable. For the getnext command the variable names are always updated and can be used to perform multiple vectored getnexts of an entire table.

## Command Examples

The following are some examples of the Exec command format.

[chart(128.9.118.88,IfPackStats.1)]

[graph(bridge1,IpStats)]

[edit(bridge,ipAddrEntry.128.9.118.77)]

[table(bridge1,ipRouteEntry)]

[run(bridge1,hubview.exe)]

[macro(router1,ranges1.mac)]

[set(bridge1,sysContact.0,"This message, has commas,")]

[set(bridge1,ifAdminStatus.1,up)]

[get(workstation,sysDescr.0,sysContact.0,sysUpTime.0)]

[getnext(workstation,ifInOctets,ifOutOctets,ifInUcastPkts)]

[set_asy(bridge1,sysDescr.0,"Hello there",sysContact.0,"Fred Spartacus")]

# DDE Errors

If an error occurs, a negative ACK message will be returned for the message. The status word contains an error code. Possible errors are tabulated below.

| Error Name | Val | Description |
| --- | --- | --- |
| NOMEM | 1 | Not enough memory |
| PROT | 3 | Read only variable |
| BADVAL | 4 | Bad value |
| BADPACK | 5 | Error while parsing SNMP frame |
| REJECT | 6 | Rejected by node |
| NET | 7 | Network error |
| TIMEOUT | 8 | Request timeout |
| BADOID | 9 | Bad Object Identifier |
| INTERNAL | 10 | Internal SNMPc error |
| ENDOFMIB | 11 | End of MIB on GetNext |
| NOFILE | 13 | Missing/bad file name |
| UNKNODE | 16 | Node name not in map |
| BADAPI | 52 | Bad DDE request format |
| BUSY | 56 | Unfinished DDE command in progress |

If a set operation succeeds, a positive ACK will be returned.

Checking Error Status with Visual Basic

When using Visual Basic the error value is not returned after each operation, although a negative Ack is.   Use get,,$error to get the error status for the last operation.

# DDELINK DLL Interface

## Overview

The DDELINK DLL provides a simple synchcronous interface to the SNMPc DDE functions.   DDELINK hides the details of communication using DDE.  However, you must be familiar with the underlying SNMPc DDE operations.

You must include “ddelink.h” to use the DDELINK DLL functions.  Each of the available DDELINK functions is described below.

## SNMPC_INIT

int snmpc_init (hInst)

HINSTANCE hInst;

The snmpc_init function initializes the DDELINK DLL for the calling task.

Parameters

hInst  -  task instance handle

Return Value

None

Comments

The snmpc_init function must be called once only, before any other DDELINK functions are called.

## SNMPC_CLOSE

int snmpc_close (hInst)

HINSTANCE hInst;

The snmpc_close function frees resources allocated to the task by the DDELINK library.

Parameters

hInst  -  task instance handle

Return Value

None

Comments

The snmpc_close function must be called before the task exits.

## SNMPC_EXEC

int snmpc_exec (hInst, hWnd, req, rows, cols,

oidvec, dest, count, timeout)

HINSTANCE hInst;

HWND hWnd;

LPSTR req;

LPINT rows;

LPINT cols;

LPSTR oidvec;

LPSTR dest;

int count;

long timeout;

The snmpc_exec function performs an SNMPc WM_DDE_EXEC command.

Parameters

hInst  - Instance of calling application

hWnd   - window handle of parent window (will automatically cancel

transaction if this window is closed)

req    - request string in the form: "[operation(node,var0,var1,var2...)]"

rows   - pointer to int to receive rows in data

cols   - pointer to int to receive cols in data

oidvec - pointer to char to receive object returned (must be 256 bytes)

dest   - pointer to char to receive data

count  - size of dest buffer in bytes

timeout  - amount of milliseconds to abort the transaction.  Usually set to 0 if you want

to use the SNMPc timeout.

Return Value

SNMPc error code.  0 means OK.

Example

#include “ddelink.h”

char oidvec[256];

char dest[1024];

int err, rows, cols;

// start a table in snmpc

err = snmpc_exec (hInst, hWnd, “[table(foonode,IfEntry.1)]”,  &rows,

&cols, oidvec, dest, sizeof(dest), 0);

// get some variables

err = snmpc_exec (hInst, hWnd, “[get(foonode,sysDescr.0,ifInOctets.1)]”,

&rows, &cols, oidvec, dest, sizeof(dest), 0);

// the variables are in a tab separated list in ‘dest’

## SNMPC_POKE

int snmpc_poke (hInst, hWnd, req, rows, cols, oidbuf, dest, count, timeout)

HINSTANCE hInst;

HWND hWnd;

LPSTR req;

LPINT rows;

LPINT cols;

LPSTR oidbuf;

LPSTR dest;

int count;

long timeout;

The snmpc_poke function performs an SNMPc WM_DDE_POKE command.

Parameters

hInst  - Instance of calling application.

hWnd   - window handle of parent window (will automatically cancel transaction if this window is closed)

req    - request string in the form: "set,node,object"

rows   - pointer to int to receive rows in data

cols   - pointer to int to receive cols in data

oidvec - pointer to char to receive object returned (must be 256 bytes)

dest   - data that will be sent to the req object

count  - ignored

timeout- amount of milliseconds to abort the transaction.  Usually set to 0 if you

want to use the SNMPc timeout.

Return Values

SNMPc error code.  0 means OK.

## SNMPC_REQUEST

int snmpc_request (hInst, hWnd, req,

rows, cols, oidbuf, dest, count, timeout)

HINSTANCE hInst;

HWND hWnd;

LPSTR req;

LPINT rows;

LPINT cols;

LPSTR oidbuf;

LPSTR dest;

int count;

long timeout;

The snmpc_request function performs an SNMPc WM_DDE_POKE operation.

Parameters

hInst  - Instance of calling application.

hWnd   - window handle of parent window (will automatically cancel

transaction if this window is closed)

req    - request string in the form: "get,node,object[,count]"

rows   - pointer to int to receive rows in data

cols   - pointer to int to receive cols in data

oidbuf - pointer to char to receive object returned (must be 256 bytes)

dest   - pointer to char to receive data in tab-separated format

count  - size of dest buffer in bytes;

timeou - amount of milliseconds to abort the transaction (0 for default).

Return Value

SNMPc error code.  0 means OK.